Click settings from the top menu bar. In order to visualize the data properly and make the auto-calibration effective is important to set the fork and shock sensor lengths. Default lengths are:

• MTB Fork 200mm

• MTB Shock 75mm

The root path defined by the user is automatically loaded at startup. Best practice is to set the root path to the main folder which contains all the .byb files.

Menu bar

• File browser

• Control panel

• Tab selector

• GPS and altitude charts

• Tabs

The control panel shows the current display mode by highliting the respective buttons.

There are several choiches which can be freely selected:

• SUSPENSION - It’s used to set the display mode to suspension travel

• WHEEL - It’s used to set the display mode to front and rear wheel travel

• TRAVEL (%) - It’s used to set the display mode to percentage

• TRAVEL (mm) - It’s used to set the dispay mode to millimiters

• LEVERAGE - It’s used to set the leverage ratio between the shock and the rear-wheel

• LAP MANAGER- It’s used to obtain time splits and lap times

The software automatically updates all the charts and visualizations inside every other tab according with the selected parameters.

Click on the “Leverage settings” button to open the leverage dialog.

Here you can set the parameters about the geometry of your frame. In particular, you can convert the shock travel into the rear-wheel travel.

The default leverage is linear and is automatically loaded from the data contained inside the notes.

The custom leverage curve is activated by filling the table.

How to plot a curve

Click on the “Lap manager” button to open the dialog.

Here you can set the “start line” and “finish line” if you are in open lap (usually used for downhill tracks), and you can set the “start/finish line” if you are in closed lap (usually used for motocross tracks).

• Insert the values of shock travel and rear-wheel travel in the table (optimal amount is at least 6/7 points). As last point use max shock travel and max rear wheel travel

• Press the “Plot” button to display the points on the graph

• To generate the curve, press the “Interpolate” button. The software automatically calculates the conversion curve relative to your swingarm. Be sure that the displayed curve is passing through the points of the graph.

• Now you can select the “Wheel travel” option from the control panel

• You can save and load the leverage points of your bikes using the “Save” and “Load” buttons

• If needed, click the “Toggle” button to remove the custom interpolation

• Select open lap or close lap, according to the type of track you’re riding.

• Draw on the map the “start line” and “finish line” or the “start/finish” line”.

• Once done press “Apply”. The stack view and suspension view will be automatically updated with

the new range, from the selected start line to the selected finish line.

• Press “Save” to save the GPS coordinates in a file. This is usefull if you want to compare exactly the same time splits or lap times among diffrent runs.

• Press “Load” to load the GPS coordinate from a previously saved file.

• Press “Toggle” to exit from the Lap visualization mode.

The software is characterized by several tabs. They are:

• STACK VIEW - It contains all the charts stacked togheter

• SUSPENSION VIEW - It containes detailed charts regarding every kind of data

• POSITION DISTRIBUTION - It contains the histograms about the bike’s balance

• VELOCITY DISTRIBUTION - It contains the histograms of the suspension’s velocities

• SPEED DISTRIBUTION - It contains the histograms of the rider’s speed

• SUMMARY - It contains all the information and numbers about the opened run

• NOTES - It contains all the settings and notes regarding the current run

• COMPARISON - This advanced function is used to compare two runs.

The GPS map is automatically loaded when a .byb file is selected.

The route travelled by the rider is painted on the map, while the dot represent the current position of the rider which is animated by the cursor that scrolls the data.

The map can be zoomed and repositioned using the arrow and mouse Wheel

The altitude chart shows the vertical drop of the track.

The steepnees is calculated from the GPS data and is approximante. Negative values represent descends while positive values represent climbs.

Stack view

14 In this window, fork travel, shock travel, GPS Speed, sensor speed, Acceleromter (IMU) and brakes are shown. It’s plotted also the comparison between the GPS and the sensor speed. The last one is characterized by a continue line and is much more responsive to sudden changes.

The IMU represents the shocks at the handlebar. This is the g-force and it’s measured with an accelerometer placed inside the acquisition unit. The value is a multiple of the acceleration of gravity.

The brakes chart can be analog or digital, according to the sensors installed.

• Select any kind of display mode from the control panel

• Scroll the charts and move the GPS pointer

• Scroll the charts and read the current value from the pointer’s labels

• Click, drag and release the mouse on any graph to zoom-in the interval. The corresponding part will be highlighted inside the GPS track too, while the average and the maximum values will be updated on the left

• In order to rescale the selection at the original size, double-click anywhere on the chart

The suspension view is very important for a deep analysis. On the left side, you can select which data to show inside the chart.

It’s included a banner which shows the time elapsed. It’s mainly use to select a particular selection and to use the system as chronometer.

Select any kind of display mode from the control panel

• Scroll the charts and move the GPS pointer

• Scroll the charts and read the current value from the labels

• Select which data to show and manage the overlay of the different lines

• Click, drag and release the mouse on any graph to zoom-in the interval. The corresponding part will be highlighted inside the GPS track too, while the average and the maximum values will be updated on the left

• The zoomed selection is shared an updated real-time between all the different tabs

• In order to rescale the selection at the original size, double-click anywhere on the chart

16 The position distribution represents for how long (in percentage) your suspension stays at a certain value of travel. This is displayed through two histograms, one for the position of the fork and one for the position of the shock. It’s a useful way to check the balance of the bike. To do this, it makes sense to plot the wheel position instead of the suspension position because wheel travel values differs a lot from shock travel values.

The chart on the right represents a dispersion of the points in which the coordinates are the front and rear travel at any time frame of the acquired run. The green line is a median which split the chart in two equal parts, while the yellow line is the best fit line for a perfectly balanced position between each wheel.

• Select any kind of display mode from the control panel

• Select which histogram to plot about the full run

• Select which histogram to plot about the current selection

• Select to sync the scatter plot with the current selection

• Select the overlay of brakes data

The velocity distribution represents for how long (in percentage) your suspension stays at a certain value of speed. This is displayed through two histograms, one for the velocity of the fork and one for the velocity of the shock. It’s a useful way to check the dyanamic balance of the bike. To do this, it makes sense to plot the wheel position instead of the suspension position because wheel speed values differs a lot from shock velocity values.

The chart on the right represents a dispersion of the points in which the coordinates are the front and rear velcoity at any time frame of the acquired run.

• Select any kind of display mode from the control panel

• Select which histogram to plot about the full run

• Select which histogram to plot about the current selection

• Select to sync the scatter plot with the current selection

• Select the overlay of brakes data

18 The speed distribution represents for how long (in percentage) a certain speed is sustained by the rider. It’s normal that the GPS speed differs from the sensor speed because the first one is less accurate.

Select to plot the histogram of the GPS speed

• Select to plot the histogram of the sensor speed

The summary shows the relevant data about the current run.

There are several data available coming from the various sensors.

20 The notes tab contains all the information, inputted by the rider/technician about the current setup.

It’s the best place to secure your settings and be able to consult them when needed.

21 This window allows you to compare and evaluate the gain/loss between two runs taken on the same track. The software automatically calculates the distance axis for both runs and displays the suspension travel in the top graph, while the gain or the loss of the first run compared with the second one is displayed in the bottom graph.

This tool is commonly used by racers and people who wants to improve their times and dig into the best set-up possible, taking into account all compromises of every section of the track.

Select any kind of display mode from the control panel

• Navigate between all the sub-tabs, such as the position, velocity and speed distributions tabs.

• Decide which vectors to plot

The auto tuning is a simple tool which uses a developed algorithm in order to find out if your suspension are working properly. The algorithm takes into account the level of the rider.

Depending on the result obtained is necessary to translate the suggestions on your bike. What to do is explained under each dot.

This function is used to export a full .pdf of the entire run. To generate the pdf gile is necessary to click on “Report” from the menu bar and select “Export report as .pdf”. This function will generate a new .pdf file that contains all the relevant information about the run.